# Crawl_Trace: Automated Detection and Tracing of Linear Landforms Using LiDAR

Version 1.0 — October 2025
Developed by Dr. M. Jaweed Nazary
School of Natural Resources, University of Missouri — Columbia
Supported by EPA Grant No. CD97790701
Contact: nazarymj@missouri.edu

## 1. Purpose

The Crawl_Trace tool automatically detects, traces, and characterizes levee-like or channel-like linear landforms from high-resolution LiDAR data. 
It processes a 1000 m × 1000 m tile centered at a user-defined location and uses a “crawling” algorithm that follows elevation gradients either uphill (levees) or downhill (channels).

## 2. Background

Traditional LiDAR-based feature identification often relies on manual inspection or simple terrain derivatives (e.g., slope, curvature). 
The Crawl_Trace algorithm provides an adaptive approach that combines random seeding, gradient following, and neighborhood analysis to detect subtle ridge-like or depression-like features.

## 3. Usage Overview

Example Python usage:

>>> traces = Crawl_Trace(
        location = (-10523769.57, 4719384.06),
        N = 100,
        min_height = 1.0,
        max_height = 50.0,
        window_size = (100.0, 5.0),
        D = 20.0,
        r = 20.0,
        resolution = 1,
        method = 1,
        random_seed = 59
    )

## 4. Parameter Guide

• **location** (tuple(float, float)): Center coordinate of the study tile in EPSG:3857 (x, y). Example: (-10523769.57, 4719384.06)

• **N** (int): Number of random seed points inside the 1000 m tile. Larger values increase coverage but runtime.

• **min_height** (float): Minimum relative height (m) for a feature to be considered valid.

• **max_height** (float): Maximum relative height (m) for candidate landforms.

• **window_size** (tuple(float, float)): Local tracing window dimensions (length, width) in meters.

• **D** (float): Lateral offset distance (m) during the crawl process.

• **r** (float): Neighborhood radius (m) used to estimate local orientation.

• **resolution** (int): LiDAR processing resolution (1, 2, 5, or 10 m).

• **method** (int): Crawling mode: 1 = uphill (levees/ridges), 2 = downhill (channels/depressions).

• **random_seed** (int): Random seed for reproducibility.

## 5. Output Description

Returns a GeoPandas GeoDataFrame of traced linear features. Each record represents a LineString feature with attributes such as:
trace_id, geometry, method, avg_height, min_height, max_height, and confidence (optional).

## 6. Recommended Workflow & Best Practices

1. Ensure input coordinates are projected in EPSG:3857 (meters).
2. Use overlapping tiles for continuous tracing across large areas.
3. Tune parameters (e.g., window_size, N, and min_height) based on feature scale.
4. Validate results visually against DEM or hillshade.
5. Compare extracted traces with reference datasets (e.g., National Levee Dataset).

## 7. Troubleshooting

- **ValueError**: Invalid parameter types — check coordinate format and thresholds.
- **No features detected**: Relax min_height or increase N.
- **Early crawl termination**: Increase r or window_size; check DEM noise.
- **PDAL failure**: Verify LiDAR availability and coordinate projection.

## 8. Citation

Nazary, M. J. (2025). Crawl_Trace: Automated Detection and Tracing of Linear Landforms Using LiDAR. 
School of Natural Resources, University of Missouri — Columbia.